VIET NAM NATIONAL UNIVERSITY HO CHI MINH CITY – UNIVERSITY OF TECHNOLOGY

FACULTY OF ELECTRICAL AND ELECTRONICS ENGINEERING

DEPARTMENT OF ELECTRONICS

🙞···☼···🙜

MILESTONE 2

COMPUTER ARCHITECTURE

DESIGN OF A SINGLE-CYCLE RISC-V PROCESSOR

Instructor: Dr. Tran Hoang Linh

| Student name | Student ID |
| --- | --- |
| Nguyen Ha Phong Thinh Dao Nguyen Khoi Nguyen Dinh Huy | 2151143 2151215 2211208 |

Ho Chi Minh, November 2025

Contents

1.	Introduction	4

2.	Block diagram of Single cycle	5

3.	Design and Verification strategy	6

3.1.	Overview	6

3.2.	ALU (alu.sv)	6

2.2.1.	Specification	6

2.2.2.	Verification plan	7

2.3.	BRU (brc.sv)	7

2.3.1.	Specification	7

2.3.2.	Verification plan	8

2.4.	ImmGen (immgen.sv)	9

2.4.1.	Specification	9

2.5.	Regfile (regfile.sv)	9

2.5.1.	Specification	9

2.5.2.	Verification plan	10

2.6.	LSU (lsu.sv)	11

2.6.1.	Specification	11

2.6.2.	Verification plan	12

2.7.	DataGen (datagen.sv)	13

2.7.1.	Specification	13

2.8.	ControlUnit (controlunit.sv)	14

2.8.1.	Specification	14

2.8.2.	Control unit table summary	17

2.8.3.	Verification plan	18

4.	Alternative Design	19

4.1.	Dedicated Load Data Extension (datagen.sv) module	19

4.2.	Hardware Multiplication (RV32M Subset)	19

4.3.	Hardware 7-Segment Display Decoder (decode_hex.sv)	19

5.	Evaluation	21

5.1.	Functional verification	21

5.2.	Applications	21

5.2.1.	Stopwatch on HEX LEDs	21

5.2.2.	Factorial Calculator on LCD	22

5.3.	Performance and Resource analysis	22

5.4.	Challenges and Limitations	22

6.	Result	23

6.1.	Quartus	23

References	24

- Introduction

In this milestone, students are tasked with designing a single-cycle RV32I processor, as discussed in lectures. The processor is implemented with the ISA RV32I, excluding FENCE instructions. To enable communication between this custom processor (soft-core) and external peripherals, some modifications to the standard processor design are necessary. The standard peripherals it must support are LCD, LEDs, and SWITCHes; seven-segment LEDs are optional. In order to accommodate the lecture material, the processor utilizes two identical logic element memory models for its memory system. One is configured as read-only for the instruction memory, while the other serves as a read-write data memory. The design must adhere to the specifications outlined in the milestone requirements and will be verified against both student-created tests and a comprehensive testbench provided on DOELAB server. While adherence to suggested specifications is not mandatory, it is strongly recommended. This report details the design strategy, verification process, results for the implemented processor, and the development of two practical applications demonstrating its capabilities: a stopwatch utilizing the seven-segment displays and a factorial calculator displaying results on the LCD.

- Block diagram of Single cycle

- Design and Verification strategy

  - Overview

The processor design adheres to the standard single-cycle RV32I architecture discussed in lectures and illustrated in Figure 1 of the Milestone 2 specification document. This architecture comprises the main functional blocks: a Program Counter (PC) with next-PC calculation logic, Instruction Memory (IS), Control Unit for instruction decoding and signal generation, Immediate Generator (ImmGen), Register File (Regfile), Arithmetic Logic Unit (ALU), Branch Comparison Unit (BRC/BRU), and a Load-Store Unit (LSU) integrating Data Memory (D$) and the I/O interface. Each instruction completes execution within a single clock cycle, progressing through the Fetch, Decode, Execute, Memory access, and Write-back stages sequentially. The LSU manages access to both the 2KiB data memory and memory-mapped peripherals according to the specified address ranges.

  - ALU (alu.sv)

    - Specification

The ALU is designed to perform the arithmetic and logical operations mandated by the RV32I ISA, encompassing both R-type and I-type instructions . These include signed addition and subtraction, unsigned multiplication, signed and unsigned set-less-than comparisons, and logical operations (and, or, xor), as well as logical and arithmetic shifts. It accepts two 32-bit operands (i_op_a, i_op_b) and a 4-bit function select code (i_alu_op), producing a 32-bit result (o_alu_data) . As required, built-in SystemVerilog operators for subtraction (-), comparison (<, >), shifting (<<, >>, >>>), and multiplication (*) were avoided in the final design implementation (used only for verification). Subtraction logic was implemented using an adder with the second operand two's complemented. The shift logic (sll, srl, sra) draws inspiration from a barrel shifter structure to efficiently handle variable shift amounts specified by the lower 5 bits of the second operand. The comparison logic (slt, sltu) employs a straightforward bitwise comparison approach, starting from the most significant bit and progressively combining results to determine the final 32-bit comparison outcome.

| Signal name | Width | Direction | Description |
| --- | --- | --- | --- |
| i_op_a | 32 | input | First operand for ALU operations or PC |
| i_op_b | 32 | input | Second operand or immediate value |
| i_alu_op | 4 | input | Operation selection |
| o_alu_data | 32 | output | Result of ALU operation |

    - Verification plan

| Title | Description |
| --- | --- |
| add/sub check | 1. Initial value for alu_op for add/sub 2. Initial random value for i_op_a & i_op_b Pass conditions: o_alu_data MUST match with Correct results |
| Set less than check | 1. Initial value for alu_op for slt 2. Initial random value for i_op_a & i_op_b Pass conditions: o_alu_data must be 1 if i_op_a < i_op_b, opposite o_alu_data = 0 |
| Set less than unsigned check | 1. Initial value for alu_op for sltu 2. Initial random value for i_op_a & i_op_b Pass conditions: o_alu_data must be 1 if i_op_a < i_op_b, opposite o_alu_data = 0 |
| AND/OR/XOR check | 1. Initial value for alu_op for and/or/xor 2. Initial random value for i_op_a & i_op_b Pass conditions: o_alu_data must be equal bitwise logic result of 2 inputs |
| Shift left logical | 1. Initial value for alu_op for sll 2. Initial random value for i_op_a & i_op_b[4:0] Pass conditions: o_alu_data must be shift left with 0 |
| Shift right logical | 1. Initial value for alu_op for srl 2. Initial random value for i_op_a & i_op_b[4:0] Pass conditions: o_alu_data must be shift right with 0 |
| Shift right arithmetic | 1. Initial value for alu_op for sra 2. Initial random value for i_op_a & i_op_b[4:0] Pass conditions: o_alu_data must be shift right with 1 |
| Multiplication check | 1. Initial value for alu_op for mul and mulhu 2. Initial random value for i_op_a & i_op_b[4:0] Pass conditions: o_alu_data MUST match with Correct results of 64-bit |

  - BRU (brc.sv)

    - Specification

The Branch Comparison Unit (referred to as BRC in the specification document) is responsible for comparing the values of two source registers (i_rs1_data, i_rs2_data) to determine the outcome for conditional branch instructions like beq, bne, blt, bge, BLTU, and bgeu . A 1-bit input signal, i_br_un, dictates whether the comparison should be signed (i_br_un = 0) or unsigned (i_br_un = 1) . The unit generates two primary 1-bit outputs: o_br_equal, which is asserted if the two operands are equal, and o_br_less, which is asserted if i_rs1_data is less than i_rs2_data according to the selected comparison mode . As per the project requirements, built-in SystemVerilog comparison operators were avoided. The comparison logic is implemented hierarchically

A base module performs a 4-bit unsigned comparison using fundamental logic gates, checking equality and determining greater/less than by examining bits from most significant to least significant.

The 32-bit unsigned comparison instantiates eight 4-bit comparators for input chunks and then uses further comparator stages including 4-bit and 2-bit to propagate and combine the results across the full 32 bits, ultimately generating the overall unsigned Eq_u, Gr_u, and Lt_u signals. Equality (o_br_equal) is derived by checking if neither Gr_u nor Lt_u is asserted.

The final signed comparison logic first checks the sign bits. If the signs differ, the result is determined immediately (negative is less than positive). If the signs are the same, the result relies directly on the outcome of the 32-bit unsigned comparison.

| Signal name | Width | Direction | Description |
| --- | --- | --- | --- |
| i_rs1_data | 32 | input | Data from source register 1 (rs1) |
| i_rs2_data | 32 | input | Data from source register 2 (rs2) |
| i_br_un | 1 | input | Select unsigned (1) / signed (0) compare |
| o_br_less | 1 | output | Output is 1 if rs1 < rs2 |
| o_br_equal | 1 | output | Output is 1 if rs1 = rs2 |

    - Verification plan

| Title | Description |
| --- | --- |
| Branch Less Unsigned check | 1. Initial value for i_rs1_data < i_rs2_data, i_br_un=1 2. Read value of o_br_less Pass condition: o_br_less = 1 |
| Branch Less Signed check | 1. Initial value for i_rs1_data < i_rs2_data, i_br_un=0 2. Read value of o_br_less Pass condition: o_br_less = 1 |
| Branch Equal Unsigned check | 1. Initial value for i_rs1_data = i_rs2_data, i_br_un=1 2. Read value of o_br_less Pass condition: o_br_equal = 1 |
| Branch Equal Signed check | 1. Initial value for i_rs1_data = i_rs2_data, i_br_un=0 2. Read value of o_br_less Pass condition: o_br_equal = 1 |

  - ImmGen (immgen.sv)

    - Specification

The Immediate Generator module is responsible for producing the correct 32-bit sign-extended immediate value based on the different RISC-V instruction formats (I, S, B, J, U). It takes the relevant bits of the 32-bit instruction (i_instr[31:7]) as input, along with a 3-bit selection signal (i_imm_sel) which determines the required immediate format. The output is the final 32-bit sign-extended immediate value (o_immext).

The internal logic uses a case statement controlled by i_imm_sel to select the appropriate bit manipulation for each instruction type:

- 3'b000 (I-Type): Sign-extends i_instr[31] 20 times and concatenates it with i_instr[31:20].

- 3'b001 (S-Type): Sign-extends i_instr[31] 20 times and concatenates it with i_instr[31:25] and i_instr[11:7].

- 3'b010 (B-Type): Sign-extends i_instr[31] 20 times and concatenates it with i_instr[7], i_instr[30:25], i_instr[11:8], and a final 0 bit.

- 3'b011 (J-Type): Sign-extends i_instr[31] 12 times and concatenates it with i_instr[19:12], i_instr[20], i_instr[30:21], and a final 0 bit.

- 3'b100 (U-Type): Concatenates i_instr[31:12] with 12 zero bits.

| Signal name | Width | Direction | Description |
| --- | --- | --- | --- |
| i_instr | 25 | input | Instruction bits [31:7] |
| i_imm_sel | 3 | input | Selects the immediate format type |
| o_immext | 32 | output | The resulting 32-bit sign-extended immediate value |

  - Regfile (regfile.sv)

    - Specification

The Register File module implements the 32 general-purpose 32-bit registers mandated by the RISC-V ISA. It provides the core storage for operand values used by the processor. Internally, it uses an array of 32 elements, each 32 bits wide.

The module features two asynchronous read ports, providing o_rs1_data and o_rs2_data based on the input addresses i_rs1_addr and i_rs2_addr, respectively. Combinational logic ensures that if either read address is 0 (referring to register x0), the corresponding output is hardwired to 32'h0, fulfilling the RISC-V requirement. Otherwise, the data is read directly from the internal register array.

A single synchronous write port allows updating a register's value. The write operation occurs on the rising edge of i_clk. If the write enable signal i_rd_wren is asserted and the destination address i_rd_addr is not 0, the data present on i_rd_data is written into the specified register. Writes targeting register x0 are explicitly ignored to maintain its zero value.

An active-low asynchronous reset (i_reset) is implemented. When asserted (low), it initializes all 32 internal registers to zero.

| Signal name | Width | Direction | Description |
| --- | --- | --- | --- |
| i_clk | 1 | input | Global clock |
| i_reset | 1 | input | Global active-low reset |
| i_rs1_addr | 5 | input | Address of the first source register (rs1) |
| i_rs2_addr | 5 | input | Address of the second source register (rs2) |
| i_rd_addr | 5 | input | Address of the destination register (rd) |
| i_rd_data | 32 | input | Data to write to the destination register |
| i_rd_wren | 1 | input | Write enable for the destination register |
| o_rs1_data | 32 | output | Data read from the first source register |
| o_rs2_data | 32 | output | Data read from the second source register |

    - Verification plan

| Title | Description |
| --- | --- |
| Read default value check | 1. Wait for reset and clock stable 2. Read value of 32 registers Pass conditions: Check value matching with default value 32'h0 |
| Read and Write value check | 1. Write random value to 32 registers 2. Read value of 32 registers Pass conditions: Check value matching with write value |
| Reset on the fly check | 1. Force the reset signal to 0 at 100ns 2. Read value of 32 registers Pass conditions: Check value matching with default value |

  - LSU (lsu.sv)

    - Specification

The Load-Store Unit serves as the interface between the processor core and the memory system, managing access to both the internal 2KiB Data Memory (D$) and Memory-Mapped I/O (MMIO) peripherals. It receives the target address (i_lsu_addr), store data (i_st_data), a write enable signal (i_lsu_wren), and the store type mask (i_bmask) from the CPU.

Address Decoding: The LSU first performs top-level address decoding using localparam constants that define the memory map . It determines whether the access targets the D$ region (0x0000_0000 to 0x0000_07FF) or one of the MMIO regions (LEDR, LEDG, HEX, LCD, Switches, Keys).

Internal D$ Implementation: For accesses within the D$ range, the LSU utilizes a specialized memory subsystem composed of the decodelsu and memory modules:

- Interleaved Memory Structure: The memory module implements the 2KiB D$ using two identical 8-bit dual-port RAMs (dual_port_ram): memeven stores bytes from even addresses (0x00, 0x02, ...), and memodd stores bytes from odd addresses (0x01, 0x03, ...) . This allows simultaneous access to potentially four consecutive bytes.

- Address Mapping and Store Logic (decodelsu): The decodelsu module takes the i_lsu_addr, i_st_data, i_bmask, and the D$-specific write enable (mem_wren = i_lsu_wren && addr_is_mem). Based on whether i_lsu_addr[0] is 0 (even) or 1 (odd), and the store type indicated by i_bmask (sb, sh, sw), decodelsu calculates the four necessary 10-bit addresses and determines which of the four RAM ports should receive write enables along with the corresponding data byte sliced from i_st_data . It handles address mapping complexities for both aligned and misaligned store operations according to its internal logic.

- Data Read and Reassembly (memory): The memory module reads four bytes concurrently from the memeven and memodd RAMs using the addresses provided by decodelsu. Critically, it then uses the LSB of the original CPU address (i_lsu_addr[0], passed as a separate input) to reorder these four bytes into a single 32-bit word (mem_ld_data) before outputting it back to the main LSU logic . This reordering ensures that mem_ld_data always represents the naturally aligned 32-bit word containing the byte(s) addressed by i_lsu_addr, regardless of whether the original address was even or odd. Reads from the underlying dual_port_rams are asynchronous.

MMIO Handling: For accesses outside the D$ range but within defined MMIO regions:

- Reads (always_comb block): The main LSU logic selects the appropriate data source for o_ld_data. If the address matches Switches (addr_is_sw) and Keys (addr_is_keys), i_io_sw and i_io_key are read. If the address matches an output peripheral (LEDR, LEDG, HEX, LCD), the current state of that peripheral's output register (o_io_ledr, etc.) is read back, applying I/O conventions for bit-width and padding. Reads to reserved areas yield zero.

- Writes (always_ff block): On the clock edge, if i_lsu_wren is high and the address targets an MMIO output register, the corresponding register (o_io_ledr, etc.) is updated synchronously. Specific logic handles bit-masking for LEDs and only allows sw (i_bmask == 3'b100) writes for the HEX displays. Writes to Switches or reserved areas are ignored.

An active-low asynchronous reset (negedge i_reset) initializes all MMIO output registers.

| Signal name | Width | Direction | Description |
| --- | --- | --- | --- |
| i_clk | 1 | input | Global clock |
| i_reset | 1 | input | Global active-low reset |
| i_lsu_addr | 32 | input | Address for read/write operation |
| i_st_data | 32 | input | Data to be stored |
| i_lsu_wren | 1 | input | Write enable (1=write, 0=read) |
| i_io_sw | 32 | input | Input data from physical switches |
| i_bmask | 3 | input | Store type mask (SW, SH, SB) |
| o_ld_data | 32 | output | Data read from D$ or MMIO register |
| o_io_ledr | 32 | output | Output register state for Red LEDs |
| o_io_ledg | 32 | output | Output register state for Green LEDs |
| o_io_hex0..7 | 7 | output | Output register states for 7-Segment displays |
| o_io_lcd | 32 | output | Output register state for LCD |

    - Verification plan

| Title | Description |
| --- | --- |
| Write and Read reverse region | 1. Write random value in reverse region 2. Read value of these reverse region Pass conditions: Check value matching with default value 32'h0 |
| Write and Read value in Data Memory | 1. Write random value in Data Memory region 2. Read value of Data Memory Pass conditions: Check value matching with write value |
| Write and Read value in LEDs and Switches regions | 1. Write random value in LEDs & SW region 2. Read value of LEDs & SW Pass conditions: Check value matching with write value |

  - DataGen (datagen.sv)

    - Specification

The Data Generator (datagen.sv) module is responsible for performing the correct sign-extension or zero-extension on data read from memory, as required by the different RISC-V load instructions (lb, lh, lw, lbu, lhu), before the final value is written back to the register file.

It takes the 32-bit data read from the Load-Store Unit (i_wb_data) and a 3-bit selection signal (i_sl_sel) as inputs. The i_sl_sel signal indicates the type of load instruction being executed. Based on this selection, the module produces the appropriately extended 32-bit output data (o_data_gen).

The internal logic uses a case statement driven by i_sl_sel:

- 3'b000 (Load Byte - lb): The least significant byte (i_wb_data[7:0]) is sign-extended to 32 bits by replicating the sign bit (i_wb_data[7]) into the upper 24 bits.

- 3'b001 (Load Half-word - lh): The least significant half-word (i_wb_data[15:0]) is sign-extended to 32 bits by replicating the sign bit (i_wb_data[15]) into the upper 16 bits.

- 3'b010 (Load Word - lw): The input data (i_wb_data) is passed directly through to the output without modification.

- 3'b011 (Load Byte Unsigned - lbu): The least significant byte (i_wb_data[7:0]) is zero-extended to 32 bits by padding the upper 24 bits with zeros.

- 3'b100 (Load Half-word Unsigned - lhu): The least significant half-word (i_wb_data[15:0]) is zero-extended to 32 bits by padding the upper 16 bits with zeros.

| Signal name | Width | Direction | Description |
| --- | --- | --- | --- |
| i_wb_data | 32 | input | Data input |
| i_sl_sel | 3 | input | Selects the type of sign/zero extension based on load type |
| o_data_gen | 32 | output | The final 32-bit data after appropriate extension |

  - ControlUnit (controlunit.sv)

    - Specification

The Control Unit serves as the processor's decoder, interpreting the incoming instruction fields (i_op, i_funct3, relevant bits of i_funct7) to generate all necessary control signals for the datapath components. It operates purely combinationally. For branch instructions, it additionally uses the comparison results (i_br_less, i_br_equal) from the BRC/BRU to determine whether the branch should be taken.

The primary function is to map each specific instruction to a unique set of control signal values, directing the flow of data and the operations performed in a single clock cycle. Below is a summary of how different instruction types are handled based on the decoded fields:

- Load Instructions (lb, lh, lw, lbu, lhu):

  - Decode based on opcode=0000011 and funct3.

  - Control Signals: Enable register write (o_rd_wren=1), select I-type immediate (o_imm_sel=000), use rs1 for ALU operand A (o_opa_sel=0), use immediate for ALU operand B (o_opb_sel=1), perform ADD in ALU for address calculation (o_alu_op=0000), disable memory write (o_mem_wren=0), select data from LSU for writeback (o_wb_sel=10), and set the correct sign/zero extension mode for DataGen (o_sl_sel varies from 000 to 100). Mark instruction as valid (o_insn_vld=1).

- Store Instructions (sb, sh, sw):

  - Decode based on opcode=0100011 and funct3.

  - Control Signals: Disable register write (o_rd_wren=0), select S-type immediate (o_imm_sel=001), use rs1 for ALU operand A (o_opa_sel=0), use immediate for ALU operand B (o_opb_sel=1), perform ADD in ALU for address calculation (o_alu_op=0000), enable memory write (o_mem_wren=1), set the store mask for LSU (o_bmask is {sw, sh, sb}). Writeback selection (o_wb_sel) is irrelevant. Mark instruction as valid (o_insn_vld=1).

- R-type Instructions (add, sub, sll, slt, sltu, xor, srl, sra, or, and, mul, mulhu):

  - Decode based on opcode=0110011 and funct3/funct7.

  - Control Signals: Enable register write (o_rd_wren=1), use rs1 for ALU operand A (o_opa_sel=0), use rs2 for ALU operand B (o_opb_sel=0), select the specific ALU operation via o_alu_op (0000 for add, 0001 for sub, 0010 for slt, 0011 for sltu, 0100 for xor, 0101 for or, 0110 for and, 0111 for sll, 1000 for slr, 1001 for sra, 1011 for mul, 1100 for mulhu), disable memory write (o_mem_wren=0), select ALU result for writeback (o_wb_sel=01). Mark instruction as valid (o_insn_vld=1).

- I-type ALU Instructions (addi, slli, slti, sltiu, xori, srli, srai, ori, andi):

  - Decode based on opcode=0010011 and funct3/funct7 (for shifts).

  - Control Signals: Enable register write (o_rd_wren=1), select I-type immediate (o_imm_sel=000), use rs1 for ALU operand A (o_opa_sel=0), use immediate for ALU operand B (o_opb_sel=1), select the specific ALU operation via o_alu_op, disable memory write (o_mem_wren=0), select ALU result for writeback (o_wb_sel=01). Mark instruction as valid (o_insn_vld=1).

- Branch Instructions (beq, bne, blt, bge, bltu, bgeu):

  - Decode based on opcode=1100011 and funct3.

  - Control Signals: Disable register write (o_rd_wren=0), select B-type immediate (o_imm_sel=010), use PC for ALU operand A (o_opa_sel=1), use immediate for ALU operand B (o_opb_sel=1), perform ADD in ALU for target address calculation (o_alu_op=0000), disable memory write (o_mem_wren=0), select the appropriate comparison mode (signed/unsigned) for BRC (o_br_un varies, 1 for signed beq/bne/blt/bge, 0 for unsigned bltu/bgeu), and set o_pc_sel to 1 only if the branch condition (based on i_br_less, i_br_equal) is met, otherwise o_pc_sel is 0. Mark instruction as valid (o_insn_vld=1).

- Jump Instructions (jal, jalr):

  - jal: Decoded by opcode=1101111. Control: Enable register write (o_rd_wren=1), select J-type immediate (o_imm_sel=011), use PC for ALU A (o_opa_sel=1), immediate for ALU B (o_opb_sel=1), ADD in ALU (o_alu_op=0000), disable memory write (o_mem_wren=0), select PC+4 for writeback (o_wb_sel=00), always take the jump (o_pc_sel=1). Valid (o_insn_vld=1).

  - jalr: Decoded by opcode=1100111 and funct3=000. Control: Enable register write (o_rd_wren=1), select I-type immediate (o_imm_sel=000), use rs1 for ALU A (o_opa_sel=0), immediate for ALU B (o_opb_sel=1), ADD in ALU (o_alu_op=0000), disable memory write (o_mem_wren=0), select PC+4 for writeback (o_wb_sel=00), always take the jump (o_pc_sel=1). Valid (o_insn_vld=1).

- U-type Instructions (lui, auipc):

  - lui: Decoded by opcode=0110111. Control: Enable register write (o_rd_wren=1), select U-type immediate (o_imm_sel=100), select immediate for ALU B (o_opb_sel=1), configure ALU to pass immediate B through (o_alu_op=1010), disable memory write (o_mem_wren=0), select ALU result for writeback (o_wb_sel=01). Valid (o_insn_vld=1).

  - auipc: Decoded by opcode=0010111. Control: Enable register write (o_rd_wren=1), select U-type immediate (o_imm_sel=100), use PC for ALU A (o_opa_sel=1), immediate for ALU B (o_opb_sel=1), perform ADD in ALU (o_alu_op=0000), disable memory write (o_mem_wren=0), select ALU result for writeback (o_wb_sel=01). Valid (o_insn_vld=1).

- Invalid/Unimplemented Instructions: If the combination of i_op, i_funct3, and i_funct7 does not match any of the implemented instructions, the Control Unit outputs a default set of safe control signals (typically disabling writes and selecting non-branch paths) and sets o_insn_vld to 0.

| Signal name | Width | Direction | Description |
| --- | --- | --- | --- |
| i_op | 7 | input | Instruction opcode field [6:0] |
| i_funct3 | 1 | input | Instruction funct3 field. |
| i_funct7_5 | 1 | input | Instruction funct7 bit [30] |
| i_funct7_0 | 1 | input | Instruction funct7 bit [25] |
| i_br_less | 1 | input | Result from BRC (rs1 < rs2) |
| i_br_equal | 1 | input | Result from BRC (rs1 == rs2) |
| o_pc_sel | 1 | output | Selects next PC (0=PC+4, 1=Branch/Jump target) |
| o_rd_wren | 1 | output | Register file write enable |
| o_br_un | 1 | output | BRC unsigned mode select |
| o_opa_sel | 1 | output | ALU operand A select (0=rs1, 1=PC) |
| o_opb_sel | 1 | output | ALU operand B select (0=rs2, 1=immediate) |
| o_mem_wren | 1 | output | LSU memory write enable |
| o_insn_vld | 1 | output | Instruction valid signal |
| o_imm_sel | 3 | output | Immediate generator format select |
| o_alu_op | 4 | output | ALU operation select |
| o_wb_sel | 2 | output | Writeback data select (00=PC+4, 01=ALU, 10=LoadData) |
| o_sl_sel | 3 | output | Sign/zero extension select for loads |
| o_bmask | 3 | output | Store type mask for LSU {sw, sh, sb} |
| o_data_sel | 1 | output | Selects data after being generated for load instructions |

    - Control unit table summary

| Instruction | Type | pc_sel | rd_wren | imm_sel[2:0] | insn_vld | br_un | opa_sel | opb_sel | alu_op | mem_wren | wb_sel[1:0] | data_sel | sl_sel |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lb rd, imm(rs1) | I | 0 | 1 | 000 | 1 | 0(-) | 0 | 1 | 0000 | 0 | 10 | 1 | 000 |  |
| lh rd, imm(rs1) | I | 0 | 1 | 000 | 1 | 0(-) | 0 | 1 | 0000 | 0 | 10 | 1 | 001 |  |
| lw rd, imm(rs1) | I | 0 | 1 | 000 | 1 | 0(-) | 0 | 1 | 0000 | 0 | 10 | 1 | 010 |  |
| lbu rd, imm(rs1) | I | 0 | 1 | 000 | 1 | 0(-) | 0 | 1 | 0000 | 0 | 10 | 1 | 011 |  |
| lhu rd, imm(rs1) | I | 0 | 1 | 000 | 1 | 0(-) | 0 | 1 | 0000 | 0 | 10 | 1 | 100 |  |
| addi rd, rs1, imm | I | 0 | 1 | 000 | 1 | 0(-) | 0 | 1 | 0000 | 0 | 01 | 0 | 000(-) |  |
| slli rd, rs1, uimm | I | 0 | 1 | 000 | 1 | 0(-) | 0 | 1 | 0111 | 0 | 01 | 0 | 000(-) |  |
| slti rd, rs1, imm | I | 0 | 1 | 000 | 1 | 0(-) | 0 | 1 | 0010 | 0 | 01 | 0 | 000(-) |  |
| sltiu rd, rs1, imm | I | 0 | 1 | 000 | 1 | 0(-) | 0 | 1 | 0011 | 0 | 01 | 0 | 000(-) |  |
| xori rd, rs1, imm | I | 0 | 1 | 000 | 1 | 0(-) | 0 | 1 | 0100 | 0 | 01 | 0 | 000(-) |  |
| srli rd, rs1, uimm | I | 0 | 1 | 000 | 1 | 0(-) | 0 | 1 | 1000 | 0 | 01 | 0 | 000(-) |  |
| srai rd, rs1, uimm | I | 0 | 1 | 000 | 1 | 0(-) | 0 | 1 | 1001 | 0 | 01 | 0 | 000(-) |  |
| ori rd, rs1, imm | I | 0 | 1 | 000 | 1 | 0(-) | 0 | 1 | 0101 | 0 | 01 | 0 | 000(-) |  |
| andi rd, rs1, imm | I | 0 | 1 | 000 | 1 | 0(-) | 0 | 1 | 0110 | 0 | 01 | 0 | 000(-) |  |
| auipc rd, upimm | U | 0 | 1 | 100 | 1 | 0(-) | 1 | 1 | 0000 | 0 | 01 | 0 | 000(-) |  |
| sb rs2, imm(rs1) | S | 0 | 0 | 001 | 1 | 0(-) | 0 | 1 | 0000 | 1 | 00(-) | 0 | 000(-) |  |
| sh rs2, imm(rs1) | S | 0 | 0 | 001 | 1 | 0(-) | 0 | 1 | 0000 | 1 | 00(-) | 0 | 000(-) |  |
| sw rs2, imm(rs1) | S | 0 | 0 | 001 | 1 | 0(-) | 0 | 1 | 0000 | 1 | 00(-) | 0 | 000(-) |  |
| add rd, rs1, rs2 | R | 0 | 1 | 000(-) | 1 | 0(-) | 0 | 0 | 0000 | 0 | 01 | 0 | 000(-) |  |
| sub rd, rs1, rs2 | R | 0 | 1 | 000(-) | 1 | 0(-) | 0 | 0 | 0001 | 0 | 01 | 0 | 000(-) |  |
| sll rd, rs1, rs2 | R | 0 | 1 | 000(-) | 1 | 0(-) | 0 | 0 | 0111 | 0 | 01 | 0 | 000(-) |  |
| slt rd, rs1, rs2 | R | 0 | 1 | 000(-) | 1 | 0(-) | 0 | 0 | 0010 | 0 | 01 | 0 | 000(-) |  |
| sltu rd, rs1, rs2 | R | 0 | 1 | 000(-) | 1 | 0(-) | 0 | 0 | 0011 | 0 | 01 | 0 | 000(-) |  |
| xor rd, rs1, rs2 | R | 0 | 1 | 000(-) | 1 | 0(-) | 0 | 0 | 0100 | 0 | 01 | 0 | 000(-) |  |
| srl rd, rs1, rs2 | R | 0 | 1 | 000(-) | 1 | 0(-) | 0 | 0 | 1000 | 0 | 01 | 0 | 000(-) |  |
| sra rd, rs1, rs2 | R | 0 | 1 | 000(-) | 1 | 0(-) | 0 | 0 | 1001 | 0 | 01 | 0 | 000(-) |  |
| or rd, rs1, rs2 | R | 0 | 1 | 000(-) | 1 | 0(-) | 0 | 0 | 0101 | 0 | 01 | 0 | 000(-) |  |
| and rd, rs1, rs2 | R | 0 | 1 | 000(-) | 1 | 0(-) | 0 | 0 | 0110 | 0 | 01 | 0 | 000(-) |  |
| mul rd, rs1, rs3 | R | 0 | 1 | 000(-) | 1 | 0(-) | 0 | 0 | 1011 | 0 | 01 | 0 | 000(-) |  |
| mulhu rd, rs1, rs4 | R | 0 | 1 | 000(-) | 1 | 0(-) | 0 | 0 | 1100 | 0 | 01 | 0 | 000(-) |  |
| lui rd, upimm | U | 0 | 1 | 100 | 1 | 0(-) | 0(-) | 1 | 1010 | 0 | 01 | 0 | 000(-) |  |
| beq rs1, rs2, label | B | 1 | 0 | 010 | 1 | 1 | 1 | 1 | 0000 | 0 | 01 | 0 | 000(-) |  |
| bne rs1, rs2, label | B | 1 | 0 | 010 | 1 | 1 | 1 | 1 | 0000 | 0 | 01 | 0 | 000(-) |  |
| blt rs1, rs2, label | B | 1 | 0 | 010 | 1 | 1 | 1 | 1 | 0000 | 0 | 01 | 0 | 000(-) |  |
| bge rs1, rs2, label | B | 1 | 0 | 010 | 1 | 1 | 1 | 1 | 0000 | 0 | 01 | 0 | 000(-) |  |
| bltu rs1, rs2, label | B | 1 | 0 | 010 | 1 | 0 | 1 | 1 | 0000 | 0 | 01 | 0 | 000(-) |  |
| bgeu rs1, rs2, label | B | 1 | 0 | 010 | 1 | 0 | 1 | 1 | 0000 | 0 | 01 | 0 | 000(-) |  |
| jalr rd, rs1, imm | I | 1 | 1 | 000 | 1 | 0(-) | 0 | 1 | 0000 | 0 | 00 | 0 | 000(-) |  |
| jal rd, label | J | 1 | 1 | 011 | 1 | 0(-) | 1 | 1 | 0000 | 0 | 00 | 0 | 000(-) |  |

    - Verification plan

| Title | Description |
| --- | --- |
| Ouput check | 1. Inital all case of op_code, funct3, funct7 2. Check all the control signals Pass conditions: All the control signals must match with Control Table |

- Alternative Design

While the core processor follows the standard single-cycle datapath, two significant modifications were implemented to enhance modularity and computational capability compared to the baseline design.

  - Dedicated Load Data Extension (datagen.sv) module

Instead of embedding the sign-extension and zero-extension logic for load instructions directly within the main datapath or the LSU, we implemented a dedicated datagen module. This module is instantiated in the datapath between the LSU's data output and the final writeback multiplexer.

- Function: It receives the 32-bit data word read from the LSU (i_wb_data) and a 3-bit control signal (o_sl_sel) from the Control Unit.

- Logic: Based on the o_sl_sel signal, which specifies the load type (lb, lh, lw, lbu, lhu), the datagen module performs the correct 32-bit sign-extension (for lb, lh) or zero-extension (for lbu, lhu) on the relevant bits of the input data. For lw, it passes the data through unmodified.

- Benefit: This approach provides better modularity and separation of concerns, simplifying the LSU and main datapath logic by isolating the load-data formatting into a single, specialized block.

  - Hardware Multiplication and Divisor (RV32M Subset)

To significantly accelerate multiplication and division operations, which would otherwise require slow software-based iterative loops, we extended the processor's capability beyond the base RV32I ISA by adding hardware support for 8 instructions from the 'M' extension: mul, mulh, mulhsu, mulhu, div, divu, rem and remu.

- ALU Modification: The Arithmetic Logic Unit (ALU) was modified to include a hardware multiplier and divider. 8 new o_alu_op codes were assigned to trigger these operations:

  - mul: Performs a signed x signed multiplication on rs1 and rs2 and returns the lower 32 bits of the result.

  - mulh: Performs a signed x signed multiplication on rs1 and rs2 and returns the upper 32 bits of the 64-bit result.

  - mulhsu: Performs a signed x unsigned multiplication on rs1 and rs2 and returns the upper 32 bits of the 64-bit result.

  - mulhu: Performs an unsigned x unsigned multiplication and returns the upper 32 bits of the 64-bit result.

  - div: Performs a signed division and returns the 32-bit quotient.

  - divu: Performs an unsigned division and returns the 32-bit quotient.

  - rem: Performs a signed remainder operation (rs1 % rs2) and returns the 32-bit remainder.

  - remu: Performs an unsigned remainder operation (rs1 % rs2) and returns the 32-bit remainder.

- Control Unit Modification: The Control Unit was updated to decode these new R-type instructions. It identifies them based on the opcode (0110011), funct3 (000 for mul, 001 for mulh, 010 for mulhsu, 011 for mulhu, 100 for div, 101 for divu, 110 for rem, 111 for remu), and the required funct7_0 bit (funct7[25]).

- Benefit: This provides a significant performance improvement for multiplication and divisor heavy tasks, as the operation completes in a single cycle instead of many.

  - Hardware 7-Segment Display Decoder (decode_hex.sv)

To simplify software development and offload computational tasks from the CPU, we implemented a significant modification to the LSU's 7-segment display logic.

- Baseline Limitation: The original specification and the baseline LSU logic required the CPU to perform all BCD-to-7-segment conversions in software. This meant the CPU had to store a lookup table in Data Memory (D$), load the correct 7-bit pattern for each digit, shift it into the correct position in a 32-bit word, and then store that final 32-bit word of patterns to the HEX MMIO address. This is instruction-intensive and consumes D$ memory.

- Alternative Design Modification: We integrated eight instances of the decode_hex.sv module directly into the lsu.sv hardware datapath (four for HEX0-3 and four for HEX4-7). The always_ff (write) logic within the LSU was modified:

- When the CPU performs a sw to 0x1000_2000, instead of writing i_st_data[6:0] directly to o_io_hex0, the LSU now takes the 4-bit binary value from i_st_data[3:0] and pipes it into the decode_hex module for HEX0.

- The 7-bit output of this decode_hex module is what drives the o_io_hex0 port.

- This is repeated for all eight HEX displays, using different 4-bit chunks from i_st_data (i_st_data[7:4] for HEX1, i_st_data[11:8] for HEX2, i_st_data[15:12] for HEX3).

- Load Operation: The load (read-back) logic remains consistent with the specification and our lsu.sv code. The always_comb block still reads the final 7-bit output signals (o_io_hex0, o_io_hex1, ...) and assembles them into a 32-bit word for o_ld_data.

- Benefit: This modification vastly simplifies software. The CPU no longer needs a lookup table or complex bit-masking/shifting to display numbers. To display "1817" on HEX3-0, the CPU can simply store the 32-bit binary value 0x01080107 directly to the 0x1000_2000 address. The hardware in the LSU automatically intercepts these binary digits (1, 8, 1, 7) and converts them into the correct 7-segment patterns for display. This offloads computation from the CPU, saves D$ memory, and simplifies assembly programming.

- Evaluation

  - Functional verification

The processor's design was rigorously verified for logical correctness using the comprehensive ISA test suite provided by the TA. Simulation results confirm that the processor passed (PASS) all test cases, verifying the correct operation of the following blocks:

- Control Unit & Datapath: Correctly decoded and executed all major instruction categories, including R-type (e.g., add, sub), I-type (addi, lw, ...), S-type (sw), B-type (beq), U-type (lui), and J-type (jal).

- ALU & BRC: All arithmetic/logic operations (including the alternative design's mul and mulhu) and branch comparison logic (beq, bne, blt, etc.) functioned as expected.

- LSU & DataGen: Load instructions (lw, lh, lbu, lb, lhu) and Store instructions (sw, sh, sb) were executed correctly. The datagen module properly handled sign-extension and zero-extension for all load variants.

An initial failure was noted in the iosw test. Debugging revealed this was caused by a mismatch between the testbench's expectation (a full 32-bit read of 0x12345678 from the driver.sv module ) and the LSU's strict implementation of the I/O convention (which zero-extended the 18-bit switch input ). After modifying the lsu.sv read logic for addr_is_sw to return the full 32-bit i_io_sw value, the test passed, confirming correct MMIO read functionality.

  - Applications

The processor's design was first rigorously verified for logical correctness. It successfully passed the comprehensive ISA test suite provided by teaching assistants, as evidenced by the simulation output. This confirmed the correct operation of all implemented instruction categories, including R-type, I-type, S-type, B-type, U-type, and J-type instructions, as well as the ALU, BRC, LSU, and other modules.

To demonstrate the processor's practical capabilities, two applications were developed in RISC-V assembly and successfully tested on the FPGA hardware.

    - Stopwatch on HEX LEDs

- Concept: A software-based stopwatch was implemented to display time in a MM:SS (Minutes:Seconds) format. The display utilizes four 7-segment displays: HEX3 (Minute-Tens), HEX2 (Minute-Ones), HEX1 (Second-Tens), and HEX0 (Second-Ones).

- Control: The application is controlled by the DE10 kit's push-buttons (KEYs), which are mapped to the LSU's I/O address 0x10010000. SW[0] functions as a toggle to start and stop the timer, while KEY[3] acts as an asynchronous reset, clearing the timer to 00:00.

- Logic:

- Simulation:

    - Factorial Calculator on LCD

- Concept: This program calculates the factorial ($N!$) of a number (N) and displays the result on the 16x2 LCD screen.

- Control: The input value N (e.g., 0-8) is read from the lower bits of the switches (SW[3:0]), which are mapped to the LSU's i_io_sw input at address 0x10010000.

- Logic:

- Simulation:

  - Performance and Resource analysis

The Timing Analyzer was executed based on the SDC1.sdc file, which constrains the input pin CLOCK_50 to a 20.000 ns period (50 MHz).

However, the resulting timing report is not a valid measure of the processor's Fmax. This is due to a critical warning:

- Warning (332060): Node: clock_25M:clk25M|o_clk was determined to be a clock but was found without an associated clock assignment.

This warning indicates that the actual internal clock driving the single_cycle core (the 25MHz output from your clock_25M module) is unconstrained. The Timing Analyzer is only analyzing paths relative to the CLOCK_50 input pin (which it also thinks is 25MHz), not the critical paths within the CPU datapath that are clocked by clock_25M:clk25M|o_clk.

The reported setup slack of +37.776 ns is therefore misleading, as it does not represent the critical path of the CPU. The report's summary confirms this:

- Info (332102): Design is not fully constrained for setup requirements

To obtain a correct Fmax, the SDC1.sdc file must be modified to (1) constrain CLOCK_50 to its true 50MHz (20ns) frequency and (2) define the clock_25M output as a create_generated_clock with a -divide_by 2 factor.

  - Challenges and Limitations

- Result

  - Quartus

| Maximum Frequency | 25 MHz |  |
| --- | --- | --- |
| Logic Utilization/Total logic elements | 19,919 | 48% |
| Total registers | 18217 |  |

References

Lastname, C. (2008). Title of the source without caps except Proper Nouns or: First word after colon. The Journal or Publication Italicized and Capped, Vol#(Issue#), Page numbers.

Lastname, O. (2010). Online journal using DOI (digital object identifier). Main Online Journal Name, Vol#(Issue#), 159-192. https://doi.org/10.1000/182

Lastname, W. (2009). Title of webpage. Site Name. Retrieved July 3, 2019, from http://www.example.com